FLARE

Fast Licensing Accident Response Engine

PWR / SMR Reactor Safety Analysis Tool

Installation & Quick-Start Guide

| Author | Robert P. Martin |
| --- | --- |
| Revision | Rev 0 — May 2026 |
| Applicability | Windows 10/11 — Python 3.11 or 3.12 |
| Code Theory Manual | FLARE_Code_Theory_Manual.docx |

# 1  Overview

FLARE (Fast Licensing Accident Response Engine) is a single-node, point-kinetics-based PWR/SMR reactor safety analysis code. It implements the thermal-hydraulic, neutronic, fuel, and radiological models required for design-basis and beyond-design-basis accident analysis under 10 CFR 50.46 and supporting regulatory guidance.

FLARE is delivered as a suite of Python modules launched through a Streamlit web interface. All tools are accessed from a single browser window; no separate installations are required beyond the initial Python environment setup described in this guide.

## 1.1  Tool Suite

| Tool / File | Description |
| --- | --- |
| flare_home.py | Navigation hub and FLARE Assistant AI chat (entry point) |
| flare_ui.py | PWR Simulator — interactive case setup, execution, and AI event narrative |
| flare_ua.py | Uncertainty Analysis — Monte Carlo BEPU sampling with AI analysis narrative |
| flare_risk.py | Risk Assessment — F-C chart per NEI 18-04 with AI risk narrative |
| flare_sim.py | Core simulation engine (called by all tools above) |
| flare_sim_batch_worker.py | Durable background worker for Run All Cases |
| flare_ua_worker.py | Durable background worker for Uncertainty Analysis runs |
| flare_risk_worker.py | Durable background worker for Risk Assessment runs |
| flare_model_editor.py | Model Editor — interactive Excel input deck creation and editing |
| flare_analyzer.py | Plant Analyzer — animated SVG primary system diagram |
| launch_flare.bat | One-click launcher (opens Streamlit + ngrok windows) |
| start_streamlit.ps1 | Activates environment and starts Streamlit on port 8501 |
| start_ngrok.ps1 | Starts ngrok tunnel for remote browser access |
| start_streamlit.ps1 | Interactive environment activation with first-time setup guide |
| start_ngrok.ps1 | Non-interactive activation used by launch_flare.bat |
| Case*_in.xlsx | Input decks (one workbook per case) |
| flare_config.txt | Optional: ANTHROPIC_API_KEY, ANTHROPIC_MODEL overrides |

## 1.2  Regulatory Basis

FLARE supports the following transient and accident categories:

- Large-Break LOCA (LBLOCA) — double-ended guillotine breaks, cold-leg and hot-leg variants

- Small-Break LOCA (SBLOCA) — break spectrum up to 10-inch equivalent diameter

- Reactivity-Initiated Accidents (RIA) — rod ejection and rod withdrawal at power and at HZP

- Anticipated Transients Without Scram (ATWS)

- Loss of Heat Sink (LOHS) — loss of SG secondary-side cooling

- Loss of Forced Circulation and pump coastdown to natural circulation

- Steam Generator Tube Rupture (SGTR) and Main Steam Line Break (MSLB)

The code theory is documented in the FLARE Code Theory Manual. Key references include 10 CFR 50.46, RG 1.203, RG 1.183, and NEI 18-04 Rev. 1.

# 2  System Requirements

| Item | Requirement |
| --- | --- |
| OS | Windows 10 or Windows 11 (64-bit) |
| Python | 3.11 or 3.12 (from python.org) — must be added to PATH |
| RAM | 4 GB minimum; 8 GB recommended for batch UA runs |
| Disk | 500 MB for environment; ~50 MB per simulation run directory |
| Network | Required only for AI narrative features (Anthropic API) and ngrok remote access |
| ngrok | Optional — required only for remote / mobile browser access |

# 3  Installation

Perform steps 3.1 through 3.5 once on a new machine. Day-to-day use is covered in Section 4.

## 3.1  Install Python

| 3.1 | Download and install Python 3.11 or 3.12 from https://www.python.org/downloads/ During installation, check the box “Add Python to PATH” before clicking Install. Verify the installation from PowerShell: python --version |
| --- | --- |

## 3.2  Create the Virtual Environment

| 3.2 | Open PowerShell and run: cd C:\Users\%USERNAME% python -m venv flare_env This creates an isolated Python environment at C:\Users\<you>\flare_env. The environment only needs to be created once. |
| --- | --- |

## 3.3  Install Required Packages

| 3.3 | Activate the new environment, then install all packages in one command: flare_env\Scripts\activate python -m pip install numpy pandas scipy openpyxl matplotlib XSteamPython plotly streamlit reportlab python-docx anthropic Package; Purpose. numpy / scipy; Numerical computing and statistical functions. pandas; Data manipulation and CSV/Excel I/O. openpyxl; Reading and writing Excel (.xlsx) input and output decks. matplotlib; Figure generation (used in non-Streamlit contexts). XSteamPython; IAPWS-IF97 water and steam properties. plotly; Interactive charts in the Streamlit UI. streamlit; Web UI framework. reportlab; PDF report generation. python-docx; Word document generation. |
| --- | --- |

## 3.4  Extract the FLARE Package

| 3.4 | Unzip the FLARE package to a folder of your choice, for example: C:\Users\<you>\Documents\FLARE\ The folder must contain flare_home.py, flare_sim.py, all flare_*.py tool files, launch_flare.bat, the two start_*.ps1 scripts, and all Case*_in.xlsx input decks. |
| --- | --- |

## 3.5  Verify the Installation

| 3.5 | Navigate to the FLARE folder and run the steady-state verification case: cd "C:\Users\<you>\Documents\FLARE" python flare_sim.py CaseSteadyState If CaseSteadyState_out.xlsx and CaseSteadyState_out.csv are produced without errors, the installation is correct. You can also check package availability: python -c "import openpyxl; import streamlit; print('OK')" |
| --- | --- |

⚠ PowerShell Execution Policy If PowerShell blocks any .ps1 script with a security error, run the following once, then retry: Unblock-File -Path ".\start_streamlit.ps1" Set-ExecutionPolicy -ExecutionPolicy RemoteSigned -Scope CurrentUser

# 4  Day-to-Day Use

## 4.1  Quick Launch (Recommended)

The easiest way to start FLARE is to double-click launch_flare.bat in the FLARE Install folder. This opens two PowerShell windows:

- Window 1 — activates the flare_env environment and starts Streamlit on port 8501

- Window 2 — starts an ngrok tunnel so FLARE can be accessed from any browser, including mobile devices

FLARE will open automatically at http://localhost:8501. The ngrok URL (printed in Window 2) can be used from any device on any network.

## 4.2  Manual Launch

If you prefer to start FLARE manually each session:

cd "C:\Users\<you>\Documents\FLARE"

.\start_streamlit.ps1

python -m streamlit run flare_home.py

The start_streamlit.ps1 script will activate the flare_env environment and start Streamlit on port 8501.

## 4.3  Running Simulations from the Command Line

Cases can also be run directly without the UI:

python flare_sim.py CaseSteadyState

python flare_sim.py CaseLBLOCA --no-figures

Output files (CaseName_out.xlsx, CaseName_out.csv) are written to the current working directory. The --no-figures flag suppresses matplotlib figure generation for faster runs.

## 4.4  AI Narrative Features

The PWR Simulator, Uncertainty Analysis, and Risk Assessment tools each include an AI narrative panel powered by the Anthropic Claude API. To enable it, create a file named flare_config.txt in the FLARE folder containing:

ANTHROPIC_API_KEY = sk-ant-...

ANTHROPIC_MODEL   = claude-sonnet-4-6

The ANTHROPIC_MODEL line is optional; it defaults to claude-sonnet-4-6 if omitted. The API key can also be set as a Windows environment variable (ANTHROPIC_API_KEY) or entered in the sidebar of the FLARE Home page.

## 4.5  ngrok Remote Access

The start_ngrok.ps1 script uses a fixed ngrok domain. If you use a different ngrok account or domain, edit the command in start_ngrok.ps1:

ngrok http 8501 --domain=your-domain.ngrok-free.dev --pooling-enabled=true

If ngrok is not installed or not required, the launch_flare.bat Window 2 can simply be closed; Streamlit continues to run on localhost:8501 regardless.

# 5  Available Cases

| Case Name | Description | Category |
| --- | --- | --- |
| CaseSteadyState | Full-power steady-state baseline | Verification |
| CaseSS | Sealed RCS at power (no SG, no break) | Verification |
| CaseNull | Null case — no heat source | Verification |
| CaseCore | Core heat transfer verification | Verification |
| CaseCoreDNBR | DNBR tracking with core model | Verification |
| CaseDNBR | DNB-initiated film boiling | DBE |
| CasePump | Pump coastdown only | AOO |
| CaseCoast | Loss of forced flow | AOO |
| CaseBlowdown | Small-break blowdown (SBLOCA) | DBE |
| CaseLBLOCA | Large-break LOCA — Bromley HTC | DBE |
| CaseLBLOCA_cold | Large-break LOCA, cold-leg model | DBE |
| CaseLBLOCA_hot | Large-break LOCA, hot-leg model | DBE |
| CaseFullLOCA | Double-ended guillotine LOCA | DBE |
| CaseECCS | ECCS injection and reflood | DBE |
| CaseMSLB | Main steam line break | DBE |
| CaseRIA | Reactivity insertion accident | DBE |
| CaseRodEject | Control rod ejection | DBE |
| CaseLOHS | Loss of heat sink | AOO/DBE |
| CaseStartup | Reactivity-driven power ascension (HZP to rated) | AOO |

Output files Each case writes CaseName_out.xlsx and CaseName_out.csv to the run directory. The Simulator UI Run All Cases mode writes each case to its own timestamped subdirectory (sim_CaseName_YYYYMMDD_HHMMSS).

# 6  Tool Reference

## 6.1  FLARE Home  (flare_home.py)

Navigation hub and entry point. Provides links to all tools, a configurable work-directory selector, and the FLARE Assistant AI chat. The FLARE Assistant uses the Anthropic Claude API with a system prompt that incorporates the Code Theory Manual, making it capable of answering detailed questions about the physics models, input parameters, and regulatory basis.

## 6.2  PWR Simulator  (flare_ui.py)

Interactive case setup and single-case or batch execution. Key features:

- Sidebar parameter overrides applied on top of the base case input deck

- Fast mode (--no-figures) toggle for rapid screening runs

- Run All Cases mode — executes all Case*_in.xlsx files via the durable batch worker (flare_sim_batch_worker.py), surviving browser disconnects

- Interactive Plotly output charts with zoom, pan, and export

- AI Event Narrative — detail-level slider (Brief / Standard / Detailed) generating a technical summary of the transient progression

## 6.3  Uncertainty Analysis  (flare_ua.py)

Monte Carlo BEPU (Best Estimate Plus Uncertainty) analysis per 10 CFR 50.46(b). Key features:

- Configurable sampling distributions read from flare_ua_variables.json (execution-time catalogue)

- Fast mode default ON; previous run reload from sidebar

- Results tab: scalar statistics (mean, std, p95, max), tornado chart of Pearson correlations

- AI Analysis Narrative — summarises the statistical spread, dominant input sensitivities, and 95th-percentile safety significance

- Durable execution via flare_ua_worker.py — runs survive browser disconnects

- Wilks criterion: N ≥ 59 for one-sided 95/95, N ≥ 93 for two-sided

## 6.4  Risk Assessment  (flare_risk.py)

Frequency-Consequence (F-C) chart tool per NEI 18-04 Rev. 1 design objective. Key features:

- Case library management and persistent frequency assignment (flare_risk_freqs.json)

- Automatic dose aggregation: D_total = D_accident + D_iodine-spike

- F-C boundary enforcement: AOO (≥10⁻²/yr, ≤1 rem), DBE (≥10⁻⁴/yr, ≤25 rem), BDBE (≥5×10⁻⁷/yr, ≤1000 rem)

- AI Risk Assessment Narrative — evaluates F-C positioning, category compliance, dose margin to limits, and PRA significance

- Durable execution via flare_risk_worker.py

## 6.5  Model Editor  (flare_model_editor.py)

Interactive Excel input deck creation and editing. Supports import and resampling of existing input files, direct cell editing of the command block, and time-series table construction. Saves to CaseName_in.xlsx in the FLARE working directory.

## 6.6  Plant Analyzer  (flare_analyzer.py)

Animated SVG diagram of the primary system that replays a saved CSV output file timestep by timestep. Visual indicators for pump status, ECCS injection, PORV state, film boiling, and rod failures. Useful for qualitative review of accident progression.

# 7  Configuration Reference

FLARE reads optional configuration from flare_config.txt in the FLARE working directory. The file uses a simple key = value format; lines beginning with # are comments.

| Key | Description / Default |
| --- | --- |
| ANTHROPIC_API_KEY | Anthropic API key for AI narrative features. Also accepted from the ANTHROPIC_API_KEY environment variable or the FLARE Home sidebar. |
| ANTHROPIC_MODEL | Claude model to use (default: claude-sonnet-4-6) |

Example flare_config.txt:

ANTHROPIC_API_KEY = sk-ant-api03-...

ANTHROPIC_MODEL   = claude-sonnet-4-6

Security note flare_config.txt contains a credential. Do not commit it to version control or share it publicly. Add it to .gitignore if the FLARE folder is a git repository.

# 8  Troubleshooting

| Symptom | Resolution |
| --- | --- |
| PowerShell blocks .ps1 scripts | Run: Set-ExecutionPolicy -ExecutionPolicy RemoteSigned -Scope CurrentUser |
| python not found after activation | Re-run start_streamlit.ps1; check flare_env\Scripts\python.exe exists |
| ModuleNotFoundError on startup | Re-run the pip install command from Step 3.3 with the environment active |
| Streamlit port already in use | Close the existing Streamlit window, or change port: python -m streamlit run flare_home.py --server.port 8502 |
| AI narrative: no API key | Add ANTHROPIC_API_KEY to flare_config.txt or set as environment variable |
| AI narrative: security flag | False positive — the tool calls api.anthropic.com with an API key. Whitelist the file or domain in your AV/EDR tool |
| OneDrive locking output files | The durable workers retry with exponential back-off. Pause OneDrive sync on the FLARE folder for long batch runs |
| Simulation output missing | Check CaseName_console.log in the run directory for the error traceback |

# 9  Notes and Known Limitations

- FLARE is a single-node (0-D) code. It cannot resolve axial temperature gradients, counter-current flow, or multi-dimensional vessel phenomena.

- The pressurizer level is a diagnostic quantity only. Surge line hydraulics and heater/spray interactions are not rigorously modelled.

- The natural circulation model is parameterised by sg_nat_frac. Actual natural circulation flow depends on loop geometry and driving head not explicitly modelled.

- The Baker-Just oxidation correlation is conservative relative to Cathcart-Pawel, which is acceptable for design-basis analysis but may overpredict ECR in beyond-DBA scenarios.

- Concurrent Risk, UA, and Run All Cases executions share the same Python process and may interfere. Run one long-running tool at a time.

- The iOS/Pyto-compatible version (flare_sim.py with reduced dependencies) is maintained separately for field use. See the Code Theory Manual for limitations.

# References

- 10 CFR 50.46  —  Acceptance Criteria for Emergency Core Cooling Systems for Light-Water Nuclear Power Reactors.

- NEI 18-04 Rev. 1 (2019)  —  Risk-Informed Performance-Based Technology-Inclusive Guidance. Nuclear Energy Institute.

- NRC Regulatory Guide 1.203 (2005)  —  Transient and Accident Analysis Methods.

- NRC Regulatory Guide 1.183 (2012)  —  Alternative Radiological Source Terms.

- FLARE Code Theory Manual, Rev 0 (2026)  —  R.P. Martin. Mathematical models, constitutive relations, and numerical methods underlying FLARE.

- FLARE Users Manual, Rev 0 (2026)  —  R.P. Martin. Intro to FLARE.

- Wilks, S.S. (1941)  —  Determination of sample sizes for setting tolerance limits. Annals of Mathematical Statistics, 12(1), 91–96.